SIMATSENGINEERING

ASSESSMENT TOOL CO2AT2

COURSE NAME: COMPUTER ARCHITECTURE 	 	COURSE CODE: CSA1221

Name: I.Raiyan	Reg.no:192521421

1.Construct a detailed concept map showing the relationship between CPU (ALU, Control Unit,

Registers), memory (cache, RAM), and I/O devices. Extend the map to include singlebus, multibus, and hierarchical bus structures, and clearly illustrate how data and control signals flow among these components during execution. Indicate possible points of delay or bottlenecks and how different bus structures help in improving performance	.

1.CPU (Central Processing Unit)

The CPU is the central processing component responsible for executing program  instructions and controlling overall system operations.

It consists of three main units: the 	Arithmetic Logic Unit (ALU)	, Control

Unit (CU) ,

and Registers.

The ALU performs arithmetic and logical operations, the Control Unit manages

instruction execution, and the Registers provide high-speed temporary storage for data and instructions.

2.Memory

The memory section includes Cache Memory , RAM, and ROM , each serving a specific purpose in data storage.

Cache Memory stores frequently accessed data to reduce memory access time, while

RAM temporarily stores active programs and data.

ROM contains permanent firmware and boot programs required for system startup.

3.I/O Devices

The concept map includes common input and output devices such as the keyboard, mouse, display, printer, and storage devices.

These peripherals enable communication between the user and the computer system.

Data is exchanged between the CPU and I/O devices through the system buses.

4.System Buses

The Address Bus carries memory and I/O addresses from the CPU to other components.

The Data Bus transfers data between the CPU, memory, and I/O devices during execution.

The Control Bus carries timing and control signals, including memory read/write and interrupt signals, to coordinate system operations.

5.Bus Architectures

The image compares Single-Bus , Multi-Bus , and Hierarchical Bus structures used in computer systems.

A Single-Bus structure is simple but suffers from bus contention, whereas a Multi-

Bus

structure enables parallel communication for improved performance.

A Hierarchical Bus structure reduces bus congestion by organizing communication into multiple levels, improving scalability and efficiency.

6.Data Flow During Execution

The CPU first checks the cache for the required instruction or data before accessing RAM.

If the data is available in the cache, it is processed immediately; otherwise, it is fetched from RAM.

After processing by the ALU, the result is stored in registers, memory, or transferred to an I/O device.

7.Control Signal Flow

The Control Unit generates control signals to manage communication between system components.

These signals include Memory Read , Memory Write	, I/O Read , I/O Write , and Interrupt Handling	.

The Control Bus carries these signals to synchronize all hardware operations during program execution.

8.Bottlenecks and Delays

The concept map identifies performance bottlenecks such as CPU speed exceeding RAM speed, bus contention, and slow I/O devices.

Heavy traffic on a single communication bus can delay data transfer between components.

These bottlenecks reduce overall system efficiency and increase execution time.

9.Performance Improvements

Cache Memory reduces memory access time by storing frequently used data close to the CPU.

Multi-Bus and Hierarchical Bus architectures improve communication speed by reducing bus contention and supporting parallel data transfer.

Additional techniques such as Direct Memory Access (DMA) , Registers, and Pipeline

Execution further enhance processing speed and system performance.

10.Overall Summary

The concept map illustrates how the CPU, Memory , and I/O Devices interact through  the Address, Data, and Control buses.

It demonstrates the advantages of different bus architectures and explains the flow of  data and control signals during instruction execution.

Finally, it highlights common system bottlenecks and the hardware techniques used to  improve computer performance, efficiency, and throughput.

2.Develop a comprehensive concept map linking the instruction execution cycle (fetch, decode, execute) with CPU performance parameters such as CPI, clock cycles, and execution time. Show how each stage contributes to overall execution time, and include connections to factors like memory access, instruction complexity, and pipeline stalls.

Also, represent how benchmarking is used to evaluate performance.

1.Instruction Execution Cycle

The concept map begins with the Instruction Execution Cycle , which consists of three stages: Fetch, Decode, and Execute .

The Fetch stage retrieves the instruction from memory, the Decode stage interpretsthe instruction and generates control signals, and the Execute stage performs the required arithmetic, logical, or memory operation.

These three stages work sequentially to complete the execution of every instruction and directly influence CPU performance.

2.CPU Performance Parameters

The concept map links the execution cycle with three important performance metrics:

CPI (Cycles Per Instruction)	, Clock Cycles, and Execution Time .

CPI represents the average number of clock cycles required to execute a single instruction, while Clock Cycles indicate the total processor cycles used during execution.

Execution Time is determined using the formula 	Execution Time = Instruction

Count

× CPI × Clock Cycle Time , making it the primary measure of processor performance.

3.Factors Affecting Performance

The image highlights three major factors that influence CPU performance:

Memory

Access, Instruction Complexity	, and Pipeline Stalls .

Memory access speed depends on cache hits and cache misses, instruction complexity determines the number of operations required, and pipeline stalls interrupt smooth instruction execution.

Together, these factors increase or decrease the number of clock cycles required, thereby affecting the total execution time.

4.Contribution of Each Stage to Execution Time

The concept map illustrates that the total execution time is the sum of Fetch Time , Decode Time , Execute Time , and additional delays.

Delays caused by cache misses, branch mispredictions, and pipeline stalls increase the overall execution time of the processor.

Efficient execution of each stage reduces CPU latency and improves processing speed.

5.Data and Control Flow During Execution

The image shows how instructions flow from 	Program Memory 	through the

Fetch,

Decode, Execute , and Write Back stages.

Data, address, and control buses enable communication between memory and the

CPU during each stage.

The Control Unit generates read, write, and interrupt signals to coordinate the complete execution process.

6.Benchmarking for Performance Evaluation

The concept map includes benchmarking as a method to evaluate and compare CPU performance.

Standard benchmarks such as 	SPEC CPU, LINPACK , Geekbench , and CoreMark are used to measure processor efficiency under different workloads.

Benchmark results provide information about execution time, throughput, CPI,

CPU utilization, and overall system performance.

7.Ways to Improve CPU Performance

The concept map presents several techniques used to improve processor performance, including Cache Memory , Instruction Pipelining , Branch Prediction , Out-ofOrder Execution , Higher Clock Frequency , and Compiler Optimization .

These techniques reduce memory access delays, minimize pipeline stalls, and increase instruction throughput.

As a result, the CPU executes instructions more efficiently, reducing execution time and improving overall system performance.

8.Overall Summary

The concept map demonstrates the relationship between the 	Instruction

Execution

Cycle, CPU Performance Parameters	, and the factors that influence execution speed.

It shows how memory access, instruction complexity, and pipeline stalls contribute to  execution time and overall processor efficiency.

Finally, it highlights the role of benchmarking and modern optimization techniques in  evaluating and improving CPU performance.

3.Create a concept map comparing 8085 and 8086 architectures, including their instruction sets, register organization, addressing modes, and memory segmentation (in

8086). Clearly show how these architectural features influence program execution, multitasking capability, and efficiency in real-time applications.

1.8085 Architecture

The concept map presents the 8085 as an 8-bit microprocessor with a simple  architecture designed for basic computing applications.

It supports a limited instruction set, 8-bit registers, and a maximum

memory space of

64 KB without memory segmentation.

Its straightforward design makes it suitable for embedded systems and small realtime control applications.

2.8086 Architecture

The 8086 is shown as a 16-bit microprocessor with a more advanced architecture than the 8085.

It includes a larger instruction set, 16-bit registers, and a 1 MB memory space using segmented memory organization.

These architectural improvements provide higher processing speed, better efficiency, and support for more complex applications.

3.Instruction Set Comparison

The concept map compares the instruction sets of both processors, showing that the

8085 supports around 74 instructions , while the 8086 supports 246 instructions .

The 8085 mainly performs byte-level operations, whereas the 8086 supports both byte and word-level operations with more powerful instructions.

A richer instruction set enables the 8086 to execute complex programs more efficiently than the 8085.

4.Register Organization

The 8085 contains 8-bit general-purpose registers (A, B, C, D, E, H, and L) and register pairs for 16-bit operations.

The 8086 contains 16-bit general-purpose registers (AX, BX, CX, DX), pointer registers, index registers, segment registers, and a flag register.

The larger register organization of the 8086 reduces memory access and improves execution speed.

5.Addressing Modes

The concept map shows that the 8085 supports basic addressing modes such as Immediate, Register, Direct, Register Indirect, and Implied.

The 8086 provides additional addressing modes including Based, Indexed, Based Indexed, and Relative addressing.

These advanced addressing modes make memory access more flexible and simplify complex program development.

6.Memory Organization and Segmentation

The 8085 uses a single continuous 64 KB memory space without segmentation.

The 8086 introduces memory segmentation	, dividing memory into Code, Data,

Stack, and Extra segments to access up to 1 MB of memory.

Memory segmentation improves memory management, program organization, and efficient utilization of available memory.

7.Impact on Program Execution

The concept map explains that the 8085 executes programs sequentially with an 8bit data path, making execution suitable for simple applications.

The 8086 uses a 16-bit data path and a Bus Interface Unit (BIU), enabling faster instruction fetching and execution.

These features allow the 8086 to execute larger and more complex programs with higher efficiency.

8.Multitasking Capability

The 8085 has limited multitasking capability because it lacks memory segmentation and advanced memory management features.

The 8086 supports better multitasking by separating code, data, and stack into different memory segments.

This architecture enables faster context switching and efficient execution of multiple tasks in operating system environments.

9.Real-Time Application Performance

The concept map compares the suitability of both processors for real-time applications.

The 8085 is ideal for simple embedded systems requiring predictable timing and low hardware complexity.

The 8086 is better suited for advanced real-time systems such as industrial automation, robotics, communication systems, and process control due to its higher speed and larger memory support.

10.Overall Summary

The concept map highlights the architectural differences between the 	8085 and

8086 in terms of instruction set, register organization, addressing modes, and memory management.

It demonstrates how these features influence program execution speed, multitasking capability, and memory utilization.

Overall, the 8085 is suitable for basic applications, whereas the 8086 provides greater performance, flexibility, and efficiency for complex and real-time computing systems.

4.Draw a detailed concept map that connects various addressing modes (immediate,

direct, register, indirect, indexed) with instruction types (data transfer, arithmetic, control

instructions). Illustrate how each addressing mode affects memory access time,

instruction size, and execution speed, and show real-world scenarios where specific addressing modes are preferred.

1.Instruction Types

The concept map begins by classifying instructions into Data Transfer , Arithmetic ,  and Control Instructions , which perform different operations during program  execution.

Data Transfer Instructions move data between registers, memory, and I/O devices,

Arithmetic Instructions perform mathematical operations, and Control Instructions

manage the execution flow of a program.

Each instruction type is linked to one or more addressing modes

depending on how  	operands are accessed.

2.Addressing Modes

The concept map illustrates five addressing modes: Immediate , Direct , Register , Indirect , and Indexed addressing.

Each addressing mode specifies a different method of locating the operand required by an instruction.

The choice of addressing mode affects memory access, instruction size, execution speed, and overall processor efficiency.

3.Immediate Addressing Mode

In Immediate Addressing , the operand is included directly within the instruction itself, eliminating the need for additional memory access.

This addressing mode provides the fastest execution speed because the CPU can use the operand immediately after fetching the instruction.

It is commonly used for loading constants, initializing variables, counters, and delay values .

4.Direct Addressing Mode

In Direct Addressing , the instruction contains the memory address of the operand.

The CPU performs one memory access to retrieve the required data, resulting in moderate execution speed.

This mode is preferred for accessing global variables, configuration data, and fixed memory locations .

5.Register Addressing Mode

In Register Addressing , the operand is stored inside a CPU register rather than in memory.

Since no external memory access is required, this mode provides very fast execution and the smallest instruction size.

It is widely used for 	high-speed calculations, loop operations, and temporary data storage.

6.Indirect Addressing Mode

In Indirect Addressing , the instruction specifies a register or memory location that contains the effective address of the operand.

Multiple memory accesses may be required, making this mode slower than immediate or register addressing.

It is suitable for pointer operations, linked lists, dynamic memory allocation, and data structures .

7.Indexed Addressing Mode

In Indexed Addressing , the effective memory address is calculated by adding an index register to a displacement value.

This addressing mode is ideal for accessing sequential data stored in arrays, tables, and strings.

It provides efficient data processing for applications involving 	array traversal, buffer management, and string manipulation	.

8.Effect on Performance

The concept map compares the impact of each addressing mode on 	memory access time, instruction size, and execution speed	.

Immediate and Register Addressing require little or no memory access and therefore provide the fastest execution, while Indirect Addressing is slower because of additional memory references.

Selecting the appropriate addressing mode improves CPU efficiency and reduces overall program execution time.

9.Real-World Scenarios

The concept map illustrates practical situations where each addressing mode is preferred in real-world applications.

Immediate addressing is used for constants, Register addressing for high-speed computation, Direct addressing for fixed data, Indirect addressing for pointers,

and

Indexed addressing for arrays and tables.

Choosing the correct addressing mode improves program performance, memory

utilization, and execution efficiency in embedded systems and generalpurpose  	computing.

10.Overall Summary

The concept map demonstrates the relationship between addressing modes,  instruction types , and processor performance .

It shows how each addressing mode influences memory access time, instruction size,  execution speed, and the selection of suitable instruction types.

Overall, the map emphasizes that selecting the appropriate addressing mode is essential  for developing efficient, fast, and optimized computer programs in both real-time and  general computing applications.

5.Prepare an elaborate concept map integrating number systems (binary and hexadecimal) with instruction representation, memory storage, and debugging processes.

Show how data is converted between formats, how instructions are stored and interpreted by the CPU, and why hexadecimal representation is commonly used in lowlevel programming and system design.

1.Number Systems

The concept map begins with the two fundamental number systems used in computing:

Binary (Base-2) and Hexadecimal (Base-16) .

Binary uses only the digits 	0 and 1, which are directly understood by digital circuits, while hexadecimal uses the symbols 	0–9 and A–F for compact representation.

These number systems form the basis for data representation, instruction encoding, and memory organization in computer systems.

2.Data Conversion Between Formats

The concept map illustrates the conversion of data between 	Decimal , Binary , and Hexadecimal formats.

Binary values are grouped into sets of four bits (nibbles), allowing direct conversion to hexadecimal without loss of information.

Data conversion enables programmers and computer systems to represent the same value in different formats for processing and analysis.

3.Instruction Representation

The concept map explains that every machine instruction consists of an 	Opcode (operation code) and an 	Operand (data or address).

Instructions are stored internally in binary format , while programmers often view and write them in hexadecimal form for better readability.

Different instruction types, such as Data Transfer , Arithmetic , Logical , Control , and

I/O Instructions , are represented using binary bit patterns.

4.Memory Storage

The memory section shows that both instructions and data are stored in memory as binary values.

Every memory location has a unique address, which is commonly represented in hexadecimal for easier identification.

The concept map demonstrates how binary data is physically stored, while hexadecimal simplifies memory visualization and programming.

5.CPU Fetch–Decode–Execute Cycle

The concept map illustrates how the CPU processes instructions through the

Fetch,

Decode, and Execute stages.

During the Fetch stage, the instruction is retrieved from memory, the Decode stage interprets the opcode and addressing mode, and the Execute stage performs the required operation using the ALU and registers.

This cycle enables the CPU to process binary instructions and produce the required output.

6.Why Hexadecimal is Commonly Used

The concept map explains that hexadecimal provides a compact and humanreadable representation of binary data.

Since one hexadecimal digit represents exactly four binary bits , it simplifiesthe reading and writing of memory addresses, instructions, and machine code.

Hexadecimal is widely used in assembly programming, system design, firmware development, debugging, and memory analysis .

7.Debugging Process

The debugging section illustrates the steps involved in identifying and correcting software or hardware errors.

It includes activities such as setting breakpoints , inspecting register values , viewing memory dumps , disassembling machine code, analyzing errors, and resuming execution.

Most debugging tools display addresses and machine code in hexadecimal because it is shorter and easier to interpret than binary.

8.Real-World Applications

The concept map highlights practical applications of binary and hexadecimal in

modern

computing systems.

These include embedded systems , 8085/8086 microprocessor programming , operating systems , device drivers , firmware development , and debugging tools .

Using	hexadecima improves programmin	efficiency,	simplifies	memory l	g

management, and supports reliable low-level system development.

9.Relationship Between Components

The concept map connects number systems , instruction representation , memory storage, CPU processing , and debugging into a continuous workflow.

Binary data is converted into hexadecimal for readability, stored in memory, interpreted by the CPU during execution, and analyzed during debugging.

These relationships demonstrate how each component contributes to efficient program execution and system operation.

10.Overall Summary

Theconceptmapintegrates binary and hexadecimal number systemswith instruction representation , memory organization , and CPU execution .

It explains how data is converted between formats, how instructions are stored and interpreted, and how hexadecimal simplifies low-level programming and debugging.

Overall, the concept map demonstrates the importance of number systems in computer architecture, memory management, processor operation, and system design.